DRAWBACKS:-

Technical & Operational Limitations

- ​CCTV Blind Spots and Obstacles: Standard classroom cameras can miss occupants sitting in corners, behind pillars, or under low camera angles, leading to false "empty room" readings.

- ​Lighting and Vision AI Failures: Poor lighting, low-resolution streams, or nighttime conditions can drastically reduce human detection accuracy (YOLO/OpenCV).

- ​Timetable Inflexibility: Static timetables do not account for impromptu study sessions, faculty meetings, club activities, or rescheduled classes, causing frequent false-positive alerts.

- ​Compute & Bandwidth Overhead: Continuously processing real-time video feeds across dozens of classrooms requires substantial GPU compute or high bandwidth if streamed to a central server.

​Hardware & Implementation Costs

- ​Sub-Metering Granularity: Most older college campuses do not have per-room sub-meters; retrofitting current sensors (CT clamps, smart breakers, ESP32 nodes) to individual switchboards adds significant hardware and rewiring costs.

- ​Baseline Energy Needs: Some appliances (servers, projectors on standby, network switches, or lab equipment) draw steady power even when rooms are empty, which can confuse simple rule-based or probabilistic models.

​Privacy & Actionability

- ​Privacy and Surveillance Concerns: Constant video analysis of classrooms can meet resistance from faculty and students regarding persistent surveillance.

- ​Lack of Automated Control: The system only notifies staff; if maintenance personnel do not manually check and switch off appliances, the wastage continues unresolved (unlike automated smart relays).

# Algorithmic & Edge Cases

# Static Object Misclassification: Vision models can mistake mannequins, large posters, human silhouettes, or stacked chairs for occupants, resulting in missed wastage detections.

# Low-Power Load Invisibility: A single left-on light tube (e.g., 20W–40W) or ceiling fan may fall within normal sensor noise or baseline fluctuations, making small-scale wastage hard to distinguish from background standby power.

# Delayed Response Latency: Periodic batch processing (e.g., running computer vision inference every 15–30 minutes to save compute) leaves a window where appliances run wastefully before an alert triggers.

# Operational & Maintenance Friction

# Notification Fatigue: If staff receive constant minor or false alerts, they will eventually ignore or mute dashboard notifications, defeating the system’s purpose.

# Camera Drift & Obstructions: Physical issues like dirty lenses, changing camera angles after maintenance, or physical obstructions (projector screens pulled down over the camera view) can break the detection pipeline without warning.

# Network Dependence & Downtime: If campus Wi-Fi drops, IoT energy monitors (like ESP32 units) and IP cameras will fail to sync with the central server, creating blind spots in real-time reporting.

# Financial & Scalability Barriers

# Diminishing ROI: The combined recurring cost of cloud storage, edge inference hardware, server maintenance, and initial IoT installations may take years to break even against the actual electricity costs saved.

# Multi-Floor Phase Distribution: Real college switchgear often splits power by phase or floor rather than individual rooms, requiring complex electrical rewiring to isolate single-classroom power signatures accurately.